This may be used when working off a forklift platform and may accompany any Work at Heights Work Permit.

| NO | Item | √ | X |
| --- | --- | --- | --- |
|  | Is the platform fitted with guardrails, intermediate rails and kickboards? | ☐ | ☐ |
|  | Is there a guard at least 2 metres high, at the back of the cage which will prevent contact with the forklift lifting mechanism? | ☐ | ☐ |
|  | Has the platform got gates with a spring loaded gate/or a latch that can be secured? | ☐ | ☐ |
|  | Is the tilt lever locked out or made inoperable? | ☐ | ☐ |
|  | Is there a fall restraint system using a full harness and short lanyard that only allows movement within the platform confines. | ☐ | ☐ |
|  | Has the forklift operator been trained in using that particular type of elevating work platform? | ☐ | ☐ |
|  | Has the machine been inspected and tested within the previous 6 months? | ☐ | ☐ |
|  | Are the operating instructions and SWL of the platform attached in a prominent position? | ☐ | ☐ |
|  | Has the platform been secured to the forks in such a way that it cannot slide or be displaced? | ☐ | ☐ |
|  | Will the machine and operator be further than 4 metres from power lines? | ☐ | ☐ |
|  | Will the machines operation create a hazard in the area? | ☐ | ☐ |
|  | Will the weight be kept to the SWL? (A person and their tools calculated at being 100kgs) | ☐ | ☐ |
|  | Is the forklift operator going to stay at the controls at all times while the platform is in use. (This is mandatory unless there is an independent means of access and egress to the platform). | ☐ | ☐ |
|  | Will all tools when used be attached to the worker by means of a rope? | ☐ | ☐ |
|  | Has an exclusion zone been roped off down below the work area? | ☐ | ☐ |

Actions arising from the check:

Checked by: …………………………………………………… Signed: …………………………………………………… Date: ………………………………