This Permit to Work Form is to be completed when undertaking blasting work. It is then to be attached to the Work Permit along with any other supporting documentation.

Starting Dates: Marking ____________________Drilling _______________________Blasting__________________

Name of Permit Issuer:  __________________________________________________________________________

Contractor Company Name: ______________________________________________________________________

Name of Permit Receiver: (Marker) ________________________________________________________________

Name of Permit Receiver: (Driller) _________________________________________________________________

Name of Permit Receiver: (Shotfirer) _______________________________________________________________

Location of Work _______________________________________________________________________________

# Daily Checks from Marking out until day of Blast

| Daily Initial Checks Checked and Ok? Day 1 2 3 4 5 6 7 |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Surface of the blast area for fractures | ☐ | ☐ | ☐ | ☐ | ☐ | ☐ | ☐ |
| The face for stability and any known faults. | ☐ | ☐ | ☐ | ☐ | ☐ | ☐ | ☐ |
| The edge protection is intact and functional. | ☐ | ☐ | ☐ | ☐ | ☐ | ☐ | ☐ |
| Access to the blast area is not compromised - all vehicles outside. | ☐ | ☐ | ☐ | ☐ | ☐ | ☐ | ☐ |
| Blast area isolated with bunds and/or cones. | ☐ | ☐ | ☐ | ☐ | ☐ | ☐ | ☐ |
| Initial of Checker on the day. |  |  |  |  |  |  |  |
| Comments |  |  |  |  |  |  |  |

# Marking Out Requirements

| Are all hazards controlled? If no, discuss in JSEA Yes No |  |  |
| --- | --- | --- |
| - Are you aware of any bench condition and face edge hazards and are they controlled? | ☐ | ☐ |
| Light vehicles must remain in the designated LV parking area. | ☐ | ☐ |
| Minimum 2m stand-off distance from the face established. (If not, QM needs to approve) | ☐ | ☐ |
| Does the stand-off distance need to be increased? To _________________ metres | ☐ | ☐ |
| Minimum stand-off distance marked with dazzle, markers, cones, fence posts? Circle which. | ☐ | ☐ |
| Safe break in bund for drill rig established? | ☐ | ☐ |
| Break in bund marked with cones. | ☐ | ☐ |
| Weather checked to avoid significant weather events. | ☐ | ☐ |
| Marking Documentation |  |  |
| Drill Plan attached. | ☐ | ☐ |
| Edge protection marked on drill plan. | ☐ | ☐ |
| Drill Plan checked and approved by Permit Issuer | ☐ | ☐ |
| Comments |  |  |

# Marking Sign Off

| Marker Name | Signature | Date |
| --- | --- | --- |
| Permit Issuer Name | Signature | Date |

# Drilling Requirements

| Are all hazards controlled? If no, discuss in JSEA. | Yes | No |
| --- | --- | --- |
| - Drill rig daily inspection sheet completed prior to start-up. | ☐ | ☐ |
| Are ground conditions suitable to prevent drill rigs toppling over. | ☐ | ☐ |
| Is the drill rig orientated to maximize distance from face? | ☐ | ☐ |
| Is the drill rig orientated to minimise toppling over? | ☐ | ☐ |
| Is drilling completed in a normal area of the quarry? Are conditions normal. | ☐ | ☐ |
| Drilling Documentation |  |  |
| Drill Plan and instructions received from the marker. | ☐ | ☐ |
| Drill log completed and attached. | ☐ | ☐ |
| Comment |  |  |

# Drilling Sign Off

| Driller Name | Signature | Date |
| --- | --- | --- |
| Permit Issuer Name | Signature | Date |

# Blasting Requirements

| Are all hazards controlled? If no, discuss in JSEA. | Yes | No |
| --- | --- | --- |
| - Staff notified of intended time/date/location/radio channel | ☐ | ☐ |
| Third parties/members of the public consulted/notified if not in clear area of at least 4x safety zone. | ☐ | ☐ |
| Access to bench clear and the Authority to Load inspection completed. | ☐ | ☐ |
| There are no known geological changes or faults that may impact on the blast? | ☐ | ☐ |
| Drill log and bore track results reviewed and discussed with driller? | ☐ | ☐ |
| Design burden was __________________________m? Minimum burden of front face holes after bore tracking is __________________m? Is the actual burden consistent with design burden? IF NOT BORE TRACKED AND YOU CANNOT ANSWER THIS, RQM SIGN OFF IS NEEDED. | ☐ | ☐ |
| Can a safety factor 4x be achieved for personnel on site when shot is in line of sight of the firing position? If no, contact the RQM for permission. | ☐ | ☐ |
| Safe location distance for personnel on site is _________________________ m |  |  |
| Safe location distance for plant/equipment/vehicles is _________________________ m |  |  |
| Firing direction understood | ☐ | ☐ |
| Have the blast monitors been set up? - How many? | ☐ | ☐ |
| Site Plan completed with and discussed with Permit Issuer. (attached on completion): |  |  |
| Blast Locations | ☐ | ☐ |
| Guard locations | ☐ | ☐ |
| Safe firing position | ☐ | ☐ |
| Exclusion zones | ☐ | ☐ |
| Are holes below or equal to their Maximum Instantaneous Design Charge? If no, note the numbers and comment | ☐ | ☐ |
| Final Checks before blasting |  |  |
| Shotfirer in the established firing position, not in line of sight of the blast. If no, must be approved by RedBull Powder Company Operations Manager and RWM | ☐ | ☐ |
| Visitors/Contractors/Personnel/Plant/Equipment on site all accounted for and in agreed safety zone. | ☐ | ☐ |
| Guards confirmed and in place to prevent accidental access to blast zone. | ☐ | ☐ |
| Post Blast Inspection |  |  |
| Fumes and dust have settled and dissipated before entering | ☐ | ☐ |
| Faces inspected and deemed safe | ☐ | ☐ |
| All holes have fired. | ☐ | ☐ |
| All clear given to proceed with normal operations | ☐ | ☐ |

# Shotfirer Sign Off

| Shotfirer Name | Signature | Date |
| --- | --- | --- |
| Permit Issuer Name | Signature | Date |

# Attachments or Tasks after completion of Permit

| Blast monitoring (i.e. noise/vibration) received and results within tolerance. | ☐ | ☐ |
| --- | --- | --- |
| Is the shot plan completed and attached? | ☐ | ☐ |
| Maximum distance rock landed from the blast? ______________m. Record in SharePoint on Rock Distances Spreadsheet |  |  |