This form may be used when working off a power elevating work platform and may accompany any work permit.

| No | Item | √ | X |
| --- | --- | --- | --- |
|  | Is the correct type of machine selected for the intended work? | ☐ | ☐ |
|  | Has the operator been trained in that particular type of elevating work platform? | ☐ | ☐ |
|  | Has the machine been inspected and tested within the previous 6 months? | ☐ | ☐ |
|  | Are all the controls clearly identified with direction of movement and operate correctly when tested? | ☐ | ☐ |
|  | Does the emergency stop operate when activated? | ☐ | ☐ |
|  | Are fuel, water, oil and battery levels adequate? | ☐ | ☐ |
|  | Hydraulic systems are free of leaks or damage? | ☐ | ☐ |
|  | Structure is free from distortion or cracking? | ☐ | ☐ |
|  | Is the machine set up on level ground with stabilisers extended to the ground? | ☐ | ☐ |
|  | Will the machine and operator be further than 4 metres from power lines? | ☐ | ☐ |
|  | Will the machine’s operation create a hazard in the area? (traffic, crushing risks etc) | ☐ | ☐ |
|  | Will the weight be kept within the SWL? (A person and their tools calculated at being 100kgs) | ☐ | ☐ |
|  | Is there a person in the open top cage (an area isolated from the edge by handrails) of a platform or at the end of a boom? (If yes, they must be attached with a safety harness and lanyard to the machine. The line should be free enough to only allow movement within the confines of the cage). | ☐ | ☐ |
|  | If a self-propelled EWP is being used check the horn operates correctly. | ☐ | ☐ |

Actions arising from the check:

Checked by: …………………………………… Signed: ………………………………………..	 Date: ………………………..