| Site: | [Site Location] | Date of Review: | [Date of Review] |
| --- | --- | --- | --- |
| Review Completed By: | [Review Completed By] |  |  |

This checklist may be used when working on rooftops and may accompany any Work at Heights Work Permit.

| NO | Item | √ | X | N/A | Comments |
| --- | --- | --- | --- | --- | --- |
|  | Has the roof structure been inspected (inside and out) to determine its strength and condition? | ☐ | ☐ | ☐ |  |
|  | Can the roof take the load of workers, materials and tools? | ☐ | ☐ | ☐ |  |
|  | Are there secure covers over skylights or roof penetrations to prevent falling through? | ☐ | ☐ | ☐ |  |
|  | Is there safe access to the roof? If not specify how this will be provided on the work permit. | ☐ | ☐ | ☐ |  |
|  | Is there a safe method of transporting tools and materials to the roof? | ☐ | ☐ | ☐ |  |
|  | Does the roof have a fall protection barrier that will support a person preventing a fall through the roof or areas where roofing will be removed? E.g. Roof mesh, tile battens or roof framing at <500mm to centre, work platform. | ☐ | ☐ | ☐ |  |
|  | Is there edge protection to prevent the risk of falling from the roof edge (work within 2 metres of the edge)? | ☐ | ☐ | ☐ |  |
|  | Is the roof pitch greater than 25 degrees? Edge protection is required. The scaffold deck must be no more than 200mm from the level edge of the roof. The max distance from the gutter line to the leading edge of the plank is 200mm, 100mm where practicable. | ☐ | ☐ | ☐ |  |
|  | Are there suitable anchor points for safety harnesses to be attached to if a fall restraint system is required? | ☐ | ☐ | ☐ |  |
|  | There is no increased risk that people may be trapped, injured or suspended at height? (Otherwise attach a suitable emergency rescue plan to the work permit.) | ☐ | ☐ | ☐ |  |
|  | Confirm footwear provides correct amount of grip for the roof surface and weather conditions. | ☐ | ☐ | ☐ |  |
|  | Is there edge protection to prevent falling material or objects from the roof? | ☐ | ☐ | ☐ |  |

Actions arising from the check:

Checked by: ………………………………..……………….. Signed: ……………………………..……………….	 Date: ………….…………